Install Crash-Resistant Recorders and Establish Flight Data Monitoring Programs

Crash Following Loss of Engine Power Due to Fuel Exhaustion

Air Methods Corporation

Eurocopter AS350 B2, N352LN

Near Mosby, Missouri

August 26, 2011

This is an excerpt from the full investigative report. This information does not include all the safety issues, findings, and recommendations issued in the report, but instead focuses on those that are directly related to the safety item. Read the complete report: https://www.ntsb.gov/investigations/AccidentReports/Reports/AAR1302.pdf

What Happened

On August 26, 2011, a Eurocopter AS350 B2 helicopter, N352LN, crashed following a loss of engine power as a result of fuel exhaustion near the Midwest National Air Center (GPH) in Mosby, Missouri. The pilot, flight nurse, flight paramedic, and patient were killed, and the helicopter was substantially damaged by impact forces. The emergency medical services (EMS) helicopter was registered to Key Equipment Finance, Inc., and operated by Air Methods Corporation, doing business as LifeNet in the Heartland, as a 14 Code of Federal Regulations (CFR) Part 135 medical flight.

The flight originated from Harrison County Community Hospital in Bethany, Missouri, and was en route to GPH to refuel. After refueling, the pilot planned to proceed to Liberty Hospital in Liberty, Missouri, which was located about 7 nautical miles (nm) from GPH. After departing from the Harrison County Community Hospital helipad, the pilot reported via radio to the AirCom day communication specialist that he had 45 minutes of fuel and four persons on board with a risk code “Blue” and was en route to GPH. While en route to GPH, the helicopter’s altitude averaged between 400 and 600 feet above ground level with an average ground speed of 111 to 116 knots. The last few minutes of data showed the helicopter in a shallow descent.

Why it Happened

Probable Cause:
The probable causes of this accident were the pilot’s failure to confirm that the helicopter had adequate fuel on board to complete the mission before making the first departure, his improper decision to continue the mission and make a second departure after he became aware of a critically low fuel level, and his failure to successfully enter an autorotation when the engine lost power due to fuel exhaustion. Contributing to the accident were (1) the pilot’s distracted attention due to personal texting during safety-critical ground and flight operations, (2) his degraded performance due to fatigue, (3) the operator’s lack of a policy requiring that an operational control center specialist be notified of abnormal fuel situations, and (4) the lack of practice representative of an actual engine failure at cruise airspeed in the pilot’s autorotation training in the accident make and model helicopter.

Loss of Control at Takeoff 
Air Methods Corporation 
Airbus Helicopters AS350 B3e, N390LG
Frisco, Colorado
July 3, 2015

This is an excerpt from the full investigative report. This information does not include all the safety issues, findings, and recommendations issued in the report, but instead focuses on those that are directly related to the safety item. Read the complete report: https://www.ntsb.gov/investigations/AccidentReports/Reports/AAR1701.pdf

What Happened

On July 3, 2015, about 1:39 p.m. mountain daylight time, an Airbus Helicopters AS350 B3e helicopter, N390LG, registered to and operated by Air Methods Corporation, lifted off from the Summit Medical Center Heliport in Frisco, Colorado, and crashed into a parking lot; the impact point was located 360 feet southwest of the ground-based helipad. The pilot was fatally injured, and the two flight nurses were seriously injured. The helicopter was destroyed by impact forces and a postcrash fire. The flight was conducted under the provisions of [Title] 14 Code of Federal Regulations Part 135 on a company flight plan. Visual meteorological conditions prevailed at the time of the accident.

Why It Happened

Probable Cause:
The probable cause of this accident was Airbus Helicopters’ dual-hydraulic AS350 B3e helicopter’s (1) preflight hydraulic check, which depleted hydraulic pressure in the tail rotor hydraulic circuit, and (2) lack of salient alerting to the pilot that hydraulic pressure was not restored before takeoff. Such alerting might have cued the pilot to his failure to reset the yaw servo hydraulic switch to its correct position during the preflight hydraulic check, which resulted in a lack of hydraulic boost to the pedal controls, high pedal forces, and a subsequent loss of control after takeoff. Contributing to the accident was the pilot’s failure to perform a hover check after liftoff, which would have alerted him to the pedal control anomaly at an altitude that could have allowed him to safely land the helicopter. Contributing to the severity of the injuries was the helicopter’s fuel system, which was not crash resistant and facilitated a fuel-fed postcrash fire.

Rapid Descent Into Terrain

Island Express Helicopters Inc.

Sikorsky S-76B, N72EX

Calabasas, California

January 26, 2020

This is an excerpt from the full investigative report. This information does not include all the safety issues, findings and recommendations issued in the report, but instead focuses on that which is directly related to the safety item. Read the complete report: https://www.ntsb.gov/investigations/AccidentReports/Reports/AAR2101.pdf

What Happened

On January 26, 2020, a Sikorsky S-76B helicopter, N72EX, entered a rapidly descending left turn and crashed into terrain in Calabasas, California. The pilot and eight passengers died, and the helicopter was destroyed. The on-demand flight was operated by Island Express Helicopters Inc. (Island Express), Long Beach, California, under visual flight rules and the provisions of Title 14 Code of Federal Regulations Part 135. The flight departed from John Wayne Airport-Orange County (SNA), Santa Ana, California, about 0907 and was destined for Camarillo Airport (CMA), Camarillo, California, about 24 miles west of the accident site.

Why It Happened

Probable Cause:
The National Transportation Safety Board determines that the probable cause of this accident was the pilot’s decision to continue flight under visual flight rules into instrument meteorological conditions, which resulted in the pilot’s spatial disorientation and loss of control. Contributing to the accident was the pilot’s likely self-induced pressure and the pilot’s plan continuation bias, which adversely affected his decision-making, and Island Express Helicopters Inc.’s inadequate review and oversight of its safety management processes.