Require and Verify the Effectiveness of Safety Management Systems in all Revenue Passenger-Carrying Aviation Operations

Collision with Terrain

Hageland Aviation Services, Inc.

dba Ravn Connect Flight 3153

Cessna 208B, N208SD

Togiak, Alaska

October 2, 2016

This is an excerpt from the full investigative report. This information does not include all the safety issues, findings, and recommendations issued in the report, but instead focuses on those that are directly related to the safety item. Read the complete report: https://www.ntsb.gov/investigations/AccidentReports/Reports/AAR1802.pdf

What Happened

On October 2, 2016, Ravn Connect flight 3153, a turbine-powered Cessna 208B Grand Caravan airplane, N208SD, collided with steep, mountainous terrain about 10 nautical miles northwest of Togiak Airport in Alaska. The two commercial pilots and the passenger were killed, and the airplane was destroyed. The scheduled commuter flight was operated under visual flight rules (VFR) by Hageland Aviation Services, Inc., Anchorage, Alaska, under the provisions of Title 14 Code of Federal Regulations (CFR) Part 135. Visual meteorological conditions (VMC) prevailed at Togiak, but a second company flight crew (whose flight departed about 2 minutes after the accident airplane and initially followed a similar route) reported that they observed unexpected fog, changing clouds, and the potential for rain along the accident route.

Flight 3153 consisted of five scheduled segments, and the accident occurred during the third segment. The first segment departed Bethel Airport in Alaska at about 9:27 a.m. and arrived at Togiak at 10:29 a.m. The second segment departed at 10:44 a.m. and arrived at Quinhagak Airport in Alaska at 11:25 a.m. While at Quinhagak, the crew unloaded cargo, boarded a passenger, and departed on the third segment for Togiak. After departure in VMC, the airplane proceeded along a generally direct route toward the destination. During the last 4 minutes of the flight, the airplane climbed as it approached the mountain ridge that it eventually struck at an elevation of about 2,300 ft msl after having likely entered instrument meteorological conditions.

Why It Happened

Probable Cause: 
The National Transportation Safety Board determines that the probable cause of this accident was the flight crew’s decision to continue the visual flight rules flight into deteriorating visibility and their failure to perform an immediate escape maneuver after entry into instrument meteorological conditions, which resulted in controlled flight into terrain (CFIT). Contributing to the accident were (1) Hageland’s allowance of routine use of the terrain inhibit switch for inhibiting the terrain awareness and warning system alerts and inadequate guidance for uninhibiting the alerts, which reduced the margin of safety, particularly in deteriorating visibility; (2) Hageland’s inadequate crew resource management (CRM) training; (3) the Federal Aviation Administration’s failure to ensure that Hageland’s approved CRM training contained all the required elements of Title 14 Code of Federal Regulations 135.330; and (4) Hageland’s CFIT-avoidance ground training, which was not tailored to the company’s operations and did not address current CFIT-avoidance technologies.

Departure From Controlled Flight

Trans-Pacific Air Charter, LLC

Learjet 35A, N452DA

Teterboro, New Jersey

May 15, 2017

This is an excerpt from the full accident report. This information does not include all the safety issues, findings, and recommendations issued in the report, but instead focuses on those that are directly related to the safety item. Read the complete report: https://www.ntsb.gov/investigations/AccidentReports/Reports/AAR1902.pdf

What Happened

On May 15, 2017, a Learjet 35A, N452DA, departed controlled flight while on a circling approach to runway 1 at Teterboro Airport in New Jersey and impacted a commercial building and parking lot. The pilot-in-command (PIC) and the second-in-command (SIC) died; no one on the ground was injured. The airplane was destroyed by impact forces and postcrash fire. The airplane was registered to A&C Big Sky Aviation, LLC, and was operated by Trans-Pacific Air Charter, LLC, under the provisions of Title 14 Code of Federal Regulations (CFR) Part 91 as a positioning flight.

Why It Happened

Probable Cause:

The probable cause of this accident was the PIC’s attempt to salvage an unstabilized visual approach, which resulted in an aerodynamic stall at low altitude. Contributing to the accident was the PIC’s decision to allow an unapproved SIC to act as PF, the PIC’s inadequate and incomplete preflight planning, and the flight crew’s lack of an approach briefing. Also contributing to the accident were Trans-Pacific’s lack of safety programs that would have enabled the company to identify and correct patterns of poor performance and procedural noncompliance and the FAA’s ineffective safety assurance system procedures, which failed to identify these company oversight deficiencies.

Inadvertent Activation of the Fuel Shutoff Lever and Subsequent Ditching

Liberty Helicopters Inc., Operating a FlyNYON Doors-Off Flight 
Airbus Helicopters AS350 B2, N350LH

New York, New York

March 11, 2018

This is an excerpt from the full accident report. This information does not include all the safety issues, findings, and recommendations issued in the report, but instead focuses on those that are directly related to the safety item. Read the complete report: https://www.ntsb.gov/investigations/AccidentReports/Reports/AAR1904.pdf

What Happened

On March 11, 2018, an Airbus Helicopters AS350 B2, N350LH, lost engine power during cruise flight, and the pilot performed an autorotative descent and ditching on the East River in New York City. The pilot sustained minor injuries, the five passengers drowned, and the helicopter was substantially damaged. The FlyNYON-branded flight was operated by Liberty Helicopters Inc. (Liberty), per a contractual agreement with NYONair; both companies considered the flight to be an aerial photography flight operated under the provisions of Title 14 Code of Federal Regulations (CFR) Part 91.

Why It Happened

Probable Cause: 
The NTSB determines the probable cause of this accident was Liberty Helicopters Inc.’s use of a NYONair-provided passenger harness/tether system, which caught on and activated the floor-mounted engine fuel shutoff lever and resulted in the in-flight loss of engine power and the subsequent ditching. Contributing to this accident were (1) Liberty’s and NYONair’s deficient safety management, which did not adequately mitigate foreseeable risks associated with the harness/tether system interfering with the floor-mounted controls and hindering passenger egress; (2) Liberty allowing NYONair to influence the operational control of Liberty’s FlyNYON flights; and (3) the Federal Aviation Administration’s inadequate oversight of Title 14 Code of Federal Regulations Part 91 revenue passenger-carrying operations. Contributing to the severity of the accident were (1) the rapid capsizing of the helicopter due to partial inflation of the emergency flotation system and (2) Liberty and NYONair’s use of the harness/tether system that hindered passenger egress.