# Eliminate Distracted Driving

Collision Between a Sport Utility Vehicle Operating with Partial Driving Automation and a Crash Attenuator

Mountain View, California

March 23, 2018

This is an excerpt from the full investigative report. This information does not include all the safety issues, findings, and recommendations issued in the report, but instead focuses on those that are directly related to the safety item. Read the complete report https://www.ntsb.gov/investigations/AccidentReports/Reports/HAR2001.pdf

What Happened

On March 23, 2018, at 9:27 a.m., a 2017 Tesla Model X P100D electric-powered sport utility vehicle (SUV), occupied by a 38-year-old male driver, was traveling south on US Highway 101 in Mountain View, Santa Clara County, California. At this location, US-101 has six southbound traffic lanes, including a high-occupancy vehicle (HOV) exit lane to State Route 85 southbound on the far left. As the SUV approached the US-101−SR-85 interchange, it was traveling in the lane second from the left, which was an HOV lane for continued travel on US-101.

While approaching a paved gore area dividing the main travel lanes of US-101 from the SR-85 left-exit ramp, the SUV moved to the left and entered the gore. The vehicle continued traveling through the gore and struck a damaged and nonoperational crash attenuator at the end of a concrete median barrier at a speed of about 71 mph. As a result of the collision, the SUV rotated counterclockwise and the front body structure separated from the rear of the vehicle. The SUV was involved in subsequent collisions with two other vehicles.

Why it Happened

Probable Cause:

The probable cause of the Mountain View, California, crash was the Tesla Autopilot system steering the sport utility vehicle into a highway gore area due to system limitations, and the driver’s lack of response due to distraction, likely from a cell phone game application and overreliance on the Autopilot partial driving automation system. Contributing to the crash was the Tesla vehicle’s ineffective monitoring of driver engagement, which facilitated the driver’s complacency and inattentiveness. Contributing to the severity of the driver’s injuries was the vehicle’s impact with a crash attenuator barrier that was damaged and nonoperational at the time of the collision due to the California Highway Patrol’s failure to report the damage following a previous crash, and systemic problems with the California Department of Transportation’s maintenance division in repairing traffic safety hardware in a timely manner.

Multivehicle Collision Interstate 44 Eastbound

Gray Summit, Missouri

August 5, 2010

This is an excerpt from the full investigative report. This information does not include all the safety issues, findings, and recommendations issued in the report, but instead focuses on those that are directly related to the safety item. Read the complete report: https://www.ntsb.gov/investigations/AccidentReports/Reports/HAR1103.pdf

What Happened

On August 5, 2010, in Gray Summit, Missouri, traffic slowed on approach to an active work zone on eastbound Interstate 44. A 2007 Volvo truck-tractor with no trailer was traveling in the right lane and had slowed or stopped behind traffic. About 10:11 a.m. central daylight time, a series of three collisions began when a 2007 GMC Sierra extended cab pickup truck merged into the right lane and struck the rear of the Volvo tractor. Two school buses from St. James High School in St. James, Missouri, were approaching the slowed traffic and the collision ahead. The lead bus was a 71-passenger bus, occupied by 23 passengers. Following closely behind the lead bus was a 72-passenger bus, occupied by 31 passengers. Seconds after the lead bus passed a motorcoach that had stopped on the shoulder, it struck the rear of the GMC pickup. This collision—the second in the series—caused the pickup to overturn onto the back of the Volvo tractor. The front of the lead bus came to rest on top of the GMC pickup and the Volvo tractor. Moments later, the following bus struck the lead bus—the third collision. As a result of this accident sequence, the driver of the GMC pickup and one passenger seated in the rear of the lead bus were killed. A total of 35 bus passengers, the 2 bus drivers, and the driver of the Volvo tractor were injured.

Why it Happened

Probable Cause:

The probable cause of the initial Gray Summit collision was distraction, likely due to a text messaging conversation being conducted by the GMC pickup driver, which resulted in his failure to notice and react to a Volvo tractor that had slowed or stopped in response to a queue that had developed in a work zone. The second collision, between the lead school bus and the GMC pickup, was the result of the bus driver’s inattention to the forward roadway due to excessive focus on a motorcoach parked on the shoulder of the road. The final collision was due to the driver of the following school bus not maintaining the recommended minimum distance from the lead school bus in the seconds preceding the accident. Contributing to the severity of the accident was the lack of forward collision warning systems on the two school buses.

Collapse of the Interstate 5 Skagit River Bridge Following a Strike by an Oversize Combination

Mount Vernon, Washington

May 23, 2013

This is an excerpt from the full investigative report. This information does not include all the safety issues, findings, and recommendations issued in the report, but instead focuses on those that are directly related to the safety item. Read the complete report: https://www.ntsb.gov/investigations/AccidentReports/Reports/HAR1401.pdf

What Happened

On Thursday, May 23, 2013, a 2010 Kenworth truck-tractor in combination with a 1997 Aspen flatbed semitrailer hauling an oversize load was traveling south on Interstate 5 near Mount Vernon, Washington. The oversize combination vehicle was being led by a pilot/escort vehicle, a 1997 Dodge Ram pickup truck. As the oversize combination vehicle traveled across the I-5 bridge above the Skagit River, its oversize load struck the bridge, damaging the structure. As a result of contact damage to the bridge’s truss structure, span 8 of the 12-span bridge collapsed into the Skagit River. Two passenger vehicles, a southbound 2010 Dodge Ram pickup truck towing a Jayco travel-trailer and a northbound 2013 Subaru XV Crosstrek, fell into the river. Eight vehicle occupants were involved in the collapse; three received minor injuries and five were uninjured.

Why it Happened

Probable Cause:

The probable cause of the Interstate 5 Skagit River Bridge span collapse was a strike to the bridge structure by an oversize combination vehicle that failed to travel in a lane with adequate overhead clearance due to deficiencies in the interdependent system of safeguards for oversize load movements. These deficiencies included (1) insufficient route planning by Mullen Trucking LP and the oversize combination vehicle driver; (2) failure of the certified pilot/escort vehicle driver to perform required duties and to communicate potential hazards, due in part to distraction caused by cell phone use; and (3) inadequate evaluation of oversize load permit requests and no provision of low-clearance warning signs in advance of the bridge by the Washington State Department of Transportation.

A gore is a boundary intended to help organize and protect traffic when vehicles enter and exit roadways.