Improve Passenger and Fishing Vessel Safety

Fire Aboard Small Passenger Vessel Conception

Platts Harbor, Channel Islands National Park, Santa Cruz Island, 21.5 miles South-Southwest of Santa Barbara, California

September 2, 2019

This is an excerpt from the full investigative report. This information does not include all the safety issues, findings and recommendations issued in the report, but instead focuses on that which is directly related to the safety item. Read the complete report https://www.ntsb.gov/investigations/AccidentReports/Reports/MAR2003.pdf

What Happened

On September 2, 2019, the US Coast Guard received a distress call from the Conception, a 75-foot-long small passenger vessel operated by Truth Aquatics, Inc. The vessel was anchored in Platts Harbor on the north side of Santa Cruz Island, 21.5 nautical miles south-southwest of Santa Barbara, California, when it caught fire. Thirty-three passengers and one crewmember died. The surviving crew were transported to shore, and two were treated for injuries. Loss of the vessel was estimated at $1.4 million.

Why it Happened

Probable Cause:

The probable cause of this accident was the failure of Truth Aquatics, Inc., to provide effective oversight of its vessel and crewmember operations, including requirements to ensure that a roving patrol was maintained, which allowed a fire of unknown cause to grow, undetected, in the vicinity of the aft salon on the main deck. Contributing to the undetected growth of the fire was the lack of a United States Coast Guard regulatory requirement for smoke detection in all accommodation spaces. Contributing to the high loss of life were the inadequate emergency escape arrangements from the vessel’s bunkroom, as both exited into a compartment that was engulfed in fire, thereby preventing escape.

Capsizing and Sinking of Fishing Vessel Destination

Bering Sea, 2.6 miles northwest of St. Georgia Island, Alaska

February 11, 2017

This is an excerpt from the full investigative brief. This information does not include all the safety issues and findings issued in the brief, but instead focuses on that which is directly related to the safety item. Read the complete brief:  https://www.ntsb.gov/investigations/AccidentReports/Reports/MAB1814.pdf

What Happened

​ On February 11, 2017, while transiting from Dutch Harbor to St. Paul Island, Alaska, to deliver bait and to fish for crab, the fishing vessel Destination capsized 2.6 miles northwest of St. George Island, Alaska, and sank several minutes later. No mayday call was received. However, a signal from the vessel’s emergency position-indicating radio beacon (EPIRB) alerted the US Coast Guard to the sinking. During search and rescue efforts, debris and an oil sheen were sighted, but none of the six crewmembers aboard were found and are thereby presumed to be dead. The value of the vessel was estimated at $2.5 million.

Why It Happened

Probable Cause:

The probable cause of the capsizing and sinking of the fishing vessel Destination was the captain’s decision to proceed during heavy freezing spray conditions without ensuring the vessel had a margin of stability to withstand an accumulation of ice or without taking sufficient mitigating action to avoid or limit the effects of icing.

Fire Aboard Roll-on/Roll-off Passenger Vessel Caribbean Fantasy

Atlantic Ocean, 2 Miles Northwest of San Juan, Puerto Rico

August 17, 2016

This is an excerpt from the full investigative report. This information does not include all the safety issues, findings and recommendations issued in the report, but instead focuses on that which is directly related to the safety item. Read the complete report https://www.ntsb.gov/investigations/AccidentReports/Reports/MAR1801.pdf

What Happened

On August 17, 2016, a fire broke out in the main engine room of the roll-on/roll-off (Ro/Ro) passenger vessel Caribbean Fantasy when fuel spraying from a leaking flange came in contact with a hot surface on the port main propulsion engine. The fire could not be contained, so the master ordered the ship to be abandoned. US Coast Guard and other first responder vessels and aircraft, along with good Samaritan vessels, helped transport all 511 passengers and crew to the port of San Juan, Puerto Rico. Several injuries, none life-threatening, occurred during firefighting and abandonment efforts. The burning vessel drifted in the wind and grounded on the sandy bottom outside the port. Three days later, the vessel was towed into the harbor, where shore-based firefighters extinguished the last of the fire. The accident resulted in an estimated $20 million in damage to the Caribbean Fantasy, which was eventually scrapped in lieu of repairs.

Why It Happened

Probable Cause:

The probable cause of the fire aboard the roll-on/roll-off passenger vessel Caribbean Fantasy was Baja Ferries’ poor safety culture and ineffective implementation of their safety management system on board the vessel, where poor maintenance practices led to an uncontained fuel spray from a blank flange at the end of the port main engine fuel supply line onto the hot exhaust manifold of the engine. Contributing to the rapid spread of the fire were fuel and lube oil quick-closing valves that were intentionally blocked open, fixed firefighting systems that were ineffective, and a structural fire boundary that failed. Contributing to the fire and the prolonged abandonment effort was the failure of the Panama Maritime Authority and the recognized organization, RINA Services, to ensure Baja Ferries’ safety management system was functional.

Capsizing and Sinking of Fishing Vessel Christopher’s Joy

Gulf of Mexico, near Southwest Pass, Louisiana

September 23, 2014

This is an excerpt from the full investigative brief. This information does not include all the safety issues and findings issued in the brief, but instead focuses on that which is directly related to the safety item. Read the complete brief:  https://www.ntsb.gov/investigations/AccidentReports/Reports/MAB1509.pdf

What Happened

​On the afternoon of September 23, 2014, the fishing vessel Christopher’s Joy capsized while trawling in the Gulf of Mexico near Southwest Pass, Louisiana. The vessel sank later that day. The master and one crewmember suffered minor lacerations, and the remaining two crewmembers are presumed dead.

Why It Happened

Probable Cause:

The probable cause of the loss of the fishing vessel Christopher’s Joy was the master’s disregard for the impact of the deployed fishing gear upon the vessel’s reserve stability while performing turning maneuvers. Contributing to the loss of life were inadequate safety training and practices.